SysTec Smart Thermostat Report

Freddy Case

Bachelor of Science in Computer Science

CS 350: Emerging System Architecture & Technology

Goran Trajkovski

4/19/2026

SysTec Smart Thermostat Report

Introduction

The objective of this report is to recommend hardware architecture for the SysTec Smart Thermostat for mass production. With the success of developing a prototype on the Raspberry Pi platform, a transition to production requires an architecture that has the right balance of reliability, cloud connectivity, and efficient memory handling. I will be comparing the Raspberry PI, Microchip, and Freescale architectures to determine the best solution for consumers looking for a smart thermostat that provides a blend ease of use and smart capabilities.

Architecture Comparison: Raspberry Pi

The Raspberry Pi architecture was ideal for the prototyping phase because of its support for using Python and its robust library support to use the peripherals GPIO, I2C, and UART (Raspberry Pi Foundation, 2020). However, as an embedded system being utilized for mass production, it allows for technical debt due to operating system overhead (Vahid et al., 2016). Because it relies on a Linux operating system, the GPIO pins are software controlled which could lead to lagging between real time button response. Also, with the Raspberry Pi architecture you will have to rely on microSD cards for flash storage which poses a risk of file system corruption if there were power fluctuations which would be unacceptable for a household utility device (Raspberry Pi Foundation, 2020). Another benefit the Raspberry Pi has is that it has Wi-Fi built in, so it has the capability to connect to the cloud (Raspberry Pi Foundation, 2020). The issue is that the cost to ship multiple computers that can run Linux on it adds cost, power draw, and maintenance of an OS system for every unit that is sold.

Architecture Comparison: Microchip

Microchip’s eXtreme Low Power (XLP) microcontroller family offers a low-cost solution for displays and provides scalability due to its capabilities to handle high resolution graphical interfaces with touch sensing (Microchip Technology, n.d.). These microcontrollers offer “on-the-fly” clock switching which allows the device to reduce frequency and save power when full power is not needed. Also, these microcontrollers have integrated LCD drivers that function in sleep mode and have dedicated hardware peripherals for I2C and GPIO. This can help reduce latency and lower the cost of production when compared to the Raspberry Pi platform. What makes Microchip stand out is their PIC32MZ-W1 and WFI32 family that are microcontroller that have Wi-Fi built in directly onto the chip (Microchip Technology, 2022). The PIC32MZ-W1 has 1 MB of flash memory, runs at 200 MHz, and has 256 KB of RAM which is enough processing power for our thermostat code, and we can add a TCP/IP stack and TLS for secure cloud connections (Microchip Technology, 2022). This microprocessor can support the same peripherals used in the prototype and the Wi-Fi module is already FCC pre-certified (Microchip Technology, 2022). It also has support for connecting to AWS, Azure, and MQTT cloud platforms out of the box which will help with cloud connectivity for SysTec (Microchip Technology, 2022).

Architecture Comparison: Freescale/NXP

The Freescale (now NXP) MC9S08LL16 architecture is optimized for thermostat applications. It supports microamp-level currents in LCD mode and features a built-in-time-of-day module and a clock triggered ADC to minimize CPU wake up time (NXP Semiconductors, 2016). One advantage of this architecture is that it can drive up to 196 LCD segments with hardware managed blinking that operates independently from the main CPU (NXP Semiconductors, 2016). The NXP architecture is designed to interface with HVAC systems latching relays, maximizing the life of the device due to it only drawing power during state change pulses (NXP Semiconductors, 2016). The downside of the MC9S08LL16 is that it’s an 8-bit chip with only 16 KB of flash and 1KB of RAM (NXP Semiconductors, 2016). That is enough for a traditional thermostat but won’t be enough for what SysTec is looking to build. On top of including the thermostat logic, adding in Wi-Fi and TLS for secure cloud communication would eat up all the memory. The LL16 also does not have Wi-Fi built in so we would have to add an external module which adds more cost per unit and uses up memory that is scarce (NXP Semiconductors, 2016).

Final Recommendation and Justification

After comparing all three architectures against the business requirements, my recommendation is to use the Microchip architecture for the Smart Thermostat.

| Requirement | Raspberry Pi | Microchip PIC32MZ-W1 | NXP MC9S08LL16 |
| --- | --- | --- | --- |
| Supports project peripherals | Yes | Yes | Yes |
| Built-in WiFi | Yes | Yes | No |
| Flash Memory | microSD based | 1MB | 16KB |
| RAM | 1GB+ | 256KB | 1KB |
| Enough Memory for production | Yes | Yes | No |

All three chips support the peripherals that are used in the prototype. However, the decision is made due to the Microchips Wi-Fi and memory usage. The Raspberry Pi has Wi-Fi but the cost to ship and maintain the OS with the amount of memory it can handle is too high for scalability. The Microchip PIC32MZ-W1 is the right fit because it hits all three requirements of supporting the peripherals used in the project, connects to the cloud via Wi-Fi, and has enough flash and RAM to support the code. This architecture is affordable to manufacture while maintaining a healthy margin, is reliable, and can connect to SysTec’s server easily which is why the Microchip PIC32MZ-W1 is the right call.

References

Microchip Technology. (n.d.). eXtreme Low Power (XLP) PIC microcontrollers.	https://www.microchip.com/

Microchip Technology. (2022). PIC32MZ1025W104 and WFI32E01 Wi-Fi MCU module data	sheet (DS70005425D). https://www.microchip.com/

NXP Semiconductors. (2016). MC9S08LL16 Series Reference Manual. https://www.nxp.com/

Raspberry Pi Foundation. (2020). Raspberry Pi 4 Model B Product Brief.	https://www.raspberrypi.org/

Vahid, F., Givargis, T., & Miller, B. (2016). CS 350: Emerging systems architectures and	technologies. Zyante Inc.